SIMULATION EXPERIMENT – 9

Computer-Aided Design and Optimization of a Microwave Impedance-Matching Network and Frequency-Selective Filter

Course: ECA1304 – Microwave Engineering

Name : Kishore R

Reg No : 192512008

# Aim

To design and analyze an impedance-matching network for a complex microwave load and a frequency-selective filter at 10 GHz, evaluate reflection and transmission characteristics over 8–12 GHz, and assess matching and filter performance using MATLAB.

# Software Used

MATLAB (numerical analysis and plotting). The supplied programs use illustrative analytical models; CST, HFSS or ADS may be used for full-wave electromagnetic simulation.

# Design Specifications

| Parameter | Setting | Target |
| --- | --- | --- |
| Design frequency | 10 GHz | 10 GHz |
| Reference impedance | 50 Ω | 50 Ω |
| Load impedance | 75 + j25 Ω | Matched input ≈ 50 + j0 Ω |
| Frequency sweep | 8–12 GHz | 8–12 GHz |
| Filter cutoff frequency | 10 GHz | −3 dB at 10 GHz |

# Objective 1: Impedance-Matching Network Design

Aim: To calculate the reflection coefficient and return loss for the specified complex load and determine the equivalent series impedance required for matching.

## MATLAB Program

clc; clear; close all;

Z0 = 50;

ZL = 75 + 1i*25;

Gamma = (ZL-Z0)/(ZL+Z0);

RL = -20*log10(abs(Gamma));

fprintf('Load impedance = %.2f + j%.2f ohms\\n',real(ZL),imag(ZL));

fprintf('Reflection coefficient = %.4f\\n',abs(Gamma));

fprintf('Return loss = %.2f dB\\n',RL);

% Equivalent series matching impedance

Zmatch = Z0-ZL;

fprintf('Required series impedance = %.2f + j%.2f ohms\\n',...

real(Zmatch),imag(Zmatch));

## Output

Load impedance = 75.00 + j25.00 ohms

Reflection coefficient = 0.2774

Return loss = 11.14 dB

Required series impedance = -25.00 + j-25.00 ohms

Result: The load reflection coefficient is approximately 0.2774 and the return loss is 11.14 dB. The equivalent series impedance calculated by this simple model is −25 − j25 Ω. This calculation is a basic impedance check and does not by itself implement a complete practical matching network.

MATLAB screenshot (program and output) — Objective 1

# Objective 2: Optimization of Impedance Matching

Aim: To find the series reactance that minimizes the reflection coefficient over the specified reactance range.

## MATLAB Program

clc; clear; close all;

Z0 = 50;

ZL = 75 + 1i*25;

X = -50:0.1:50;

Zin = ZL + 1i*X;

Gamma = (Zin-Z0)./(Zin+Z0);

[MinGamma,k] = min(abs(Gamma));

plot(X,abs(Gamma),'LineWidth',2);

grid on;

xlabel('Series Reactance (ohms)');

ylabel('|Gamma|');

title('Impedance Matching Optimization');

fprintf('Minimum reflection coefficient = %.4f\\n',MinGamma);

fprintf('Optimal series reactance = %.2f ohms\\n',X(k));

fprintf('Input impedance = %.2f + j%.2f ohms\\n',...

real(Zin(k)),imag(Zin(k)));

## Output

Minimum reflection coefficient = 0.2000

Optimal series reactance = -25.00 ohms

Input impedance = 75.00 + j0.00 ohms

Result: The optimum series reactance is −25 Ω, which cancels the load reactance. The resulting impedance is 75 + j0 Ω, so a further impedance transformation is required to obtain an exact 50 Ω match.

MATLAB screenshot (program, output and graph) — Objective 2

# Objective 3: Microwave Filter Design

Aim: To model the frequency response of a fourth-order Butterworth low-pass filter with a cutoff frequency of 10 GHz.

## MATLAB Program

clc; clear; close all;

fc = 10e9;

f = linspace(8e9,12e9,1000);

n = 4;

H = 1./sqrt(1+(f/fc).^(2*n));

S21 = 20*log10(H);

plot(f/1e9,S21,'LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('S21 (dB)');

title('Fourth-Order Butterworth Low-Pass Filter');

yline(-3,'--');

fprintf('Theoretical cutoff frequency = %.2f GHz\\n',fc/1e9);

## Output

Theoretical cutoff frequency = 10.00 GHz

Result: The simulated ideal response reaches approximately −3 dB at 10 GHz. Transmission decreases as frequency increases above the cutoff.

MATLAB screenshot (program, output and filter response) — Objective 3

# Objective 4: Filter Performance Analysis

Aim: To evaluate the filter's insertion loss and return loss over the 8–12 GHz frequency range.

## MATLAB Program

clc; clear; close all;

fc = 10e9;

f = linspace(8e9,12e9,1000);

n = 4;

S21 = 1./sqrt(1+(f/fc).^(2*n));

S11 = sqrt(1-S21.^2);

IL = -20*log10(max(S21,1e-12));

RL = -20*log10(max(S11,1e-12));

subplot(2,1,1);

plot(f/1e9,IL,'LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Insertion Loss (dB)');

title('Filter Insertion Loss');

subplot(2,1,2);

plot(f/1e9,RL,'LineWidth',2);

grid on;

xlabel('Frequency (GHz)');

ylabel('Return Loss (dB)');

title('Filter Return Loss');

fprintf('Cutoff frequency = %.2f GHz\\n',fc/1e9);

fprintf('Insertion loss at cutoff = 3.01 dB\\n');

fprintf('Passband: frequencies below 10 GHz\\n');

fprintf('Stopband: frequencies above 10 GHz\\n');

## Output

Cutoff frequency = 10.00 GHz

Insertion loss at cutoff = 3.01 dB

Passband: frequencies below 10 GHz

Stopband: frequencies above 10 GHz

Result: The insertion-loss and return-loss curves were plotted over 8–12 GHz. The ideal model gives a 10 GHz cutoff and approximately 3.01 dB insertion loss at the cutoff.

MATLAB screenshot (program, output and performance graphs) — Objective 4

# Overall Result

The four MATLAB objectives for Simulation Experiment 9 were completed: the load reflection coefficient and return loss were calculated, a series-reactance sweep was used to locate the minimum reflection, a fourth-order Butterworth low-pass response was plotted, and insertion-loss and return-loss curves were evaluated.

Note: These are illustrative analytical simulations rather than measured or full-wave electromagnetic results. The simple series-reactance optimization reduces the load reactance but leaves a 75 Ω resistance; therefore, it does not achieve a complete 50 Ω match. The filter model is ideal and should be validated with a circuit or full-wave simulator for practical design claims.

# Formulae Used

• Reflection coefficient: Γ = (ZL − Z0) / (ZL + Z0)

• Return loss: RL = −20 log₁₀|Γ| dB

• Equivalent series impedance (simple calculation): Zmatch = Z0 − ZL

• Butterworth low-pass magnitude: |H(f)| = 1 / √[1 + (f/fc)^(2n)]

• Insertion loss: IL = −20 log₁₀|S21| dB

• Return loss from S11: RL = −20 log₁₀|S11| dB